CODE:

import RPi.GPIO as GPIO

import time

import os

import glob

# Wire for DS18B20

os.system('modprobe w1-gpio')

os.system('modprobe w1-therm')

# Pins (BOARD numbering)

TRIG = 11

ECHO = 12

BUZZER = 13

VIBRATOR = 15

#Get all the filenames begin with 28 in the path base_dir

base_dir = '/sys/bus/w1/devices/'

device_folder = glob.glob(base_dir + '28*')[0]

device_file = device_folder + '/w1_slave'

# Vibrator logic

ACTIVE_LOW_VIBRATOR = False

V_ON  = GPIO.LOW  if ACTIVE_LOW_VIBRATOR else GPIO.HIGH

V_OFF = GPIO.HIGH if ACTIVE_LOW_VIBRATOR else GPIO.LOW

def setup():

#Setup the GPIO pins for Buzzer,Vibrator, and Ultrasonic Sensor

GPIO.setmode(GPIO.BOARD)

# Ultrasonic

GPIO.setup(TRIG, GPIO.OUT)

GPIO.setup(ECHO, GPIO.IN)

# Outputs

GPIO.setup(BUZZER, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(VIBRATOR, GPIO.OUT, initial=V_OFF)

def distance():

# Measure the distance with ultrasonic sensor

GPIO.output(TRIG, 0)

time.sleep(0.000002)

GPIO.output(TRIG, 1)

time.sleep(0.00001)

GPIO.output(TRIG, 0)

while GPIO.input(ECHO) == 0:

pass

time1 = time.time()

while GPIO.input(ECHO) == 1:

pass

time2 = time.time()

duration = time2 - time1

return (duration * 340 / 2) * 100  # Convert to centimeters

def buzzer_on():

# Set buzzer on

GPIO.output(BUZZER, GPIO.LOW)

def buzzer_off():

# Set buzzer of

GPIO.output(BUZZER, GPIO.HIGH)

def vibrator_on():

# Set vibrator on

GPIO.output(VIBRATOR, V_ON)

def vibrator_off():

# Set vibrator off

GPIO.output(VIBRATOR, V_OFF)

def beep(duration):

# Set the beep for a specified duration

buzzer_on()

time.sleep(duration)

buzzer_off()

time.sleep(duration)

#For temperature sensor

def read_rom():

name_file=device_folder+'/name'

f = open(name_file,'r')

return f.readline()

def read_temp_raw():

f = open(device_file, 'r')

lines = f.readlines()

f.close()

return lines

def read_temp():

lines = read_temp_raw()

#Analyze if the last 3 characters are "YES"

while lines[0].strip()[-3:] != 'YES':

time.sleep(0.2)

lines = read_temp_raw()

#Fix the index of t= in the string

equals_pos = lines[1].find('t=')

if equals_pos != -1:

temp_string = lines[1][equals_pos+2:]

temp_c = float(temp_string) / 1000.0

temp_f = temp_c * 9.0 / 5.0 + 32.0

return temp_c, temp_f

def loop():

# Main loop that checks the distance,temperature and controls the buzzer and vibrator

while True:

dis = distance()

temp_celsius , temp_fahrenheit = read_temp()

print(dis, 'cm')  # Print distance measurement

print(f"temp: {temp_fahrenheit}")

if dis < 10:  # If the object is within 5 cm, vibrate continuously

vibrator_on()

else:

# Turn off vibrator if object is far

vibrator_off()

if temp_fahrenheit > 120:  # If the object is within 5 cm, buzz continuously

buzzer_on()

elif temp_fahrenheit < 100 and temp_fahrenheit > 80 :  # If within 100 and 120 fahrenheit, beep with decreasing interval

beep_interval = (temp_fahrenheit - 60) / 50.0  # Adjust beep interval

beep(beep_interval)

else:

buzzer_off()  # Turn off buzzer if object is far

time.sleep(0.3)

def destroy():

# Cleanup function to reset GPIO settings

GPIO.cleanup()

if __name__ == "__main__":

setup()

try:

loop()

except KeyboardInterrupt:

destroy()